AfCHIX Workshop, October 2010, Nairobi, Kenya

Track SA-E - More Networking

practice: ping, netstat, tcpdump, traceroute, arp, route

*** NOTE: These exercises should be carried out as the 'root' user ***

1.  Remember to check your network configuration!

* Check it with:

# ifconfig em0 inet

-> Do you see an IP address on your network card ?

It should look like this:

em0: flags=8843<UP,BROADCAST,RUNNING,SIMPLEX,MULTICAST> metric 0 mtu 1500

options=8<VLAN_MTU>

inet 41.215.76.x netmask 0xffffff00 broadcast 41.215.76.255

... where 'x' is your IP

* If you em0 netcard does not have a 41.215.76.x IP, then configure it:

# ifconfig em0 41.215.76.x/24

# route add default 41.215.76.254

* Remember to add your IPv6 address:

# ifconfig em0 inet6 2001:470:933d::X

# route add -inet6 default  2001:470:933d::254

* Check the network configuration again with ifconfig

em0: flags=8843<UP,BROADCAST,RUNNING,SIMPLEX,MULTICAST> metric 0 mtu 1500

options=19b<RXCSUM,TXCSUM,VLAN_MTU,VLAN_HWTAGGING,VLAN_HWCSUM,TSO4>

ether 00:1e:0b:b5:a3:c9

inet 41.215.76.X netmask 0xffffff00 broadcast 41.215.76.255

inet6 2001:470:933d::X prefixlen 64

media: Ethernet autoselect (10baseT/UTP <full-duplex>)

status: active

* Additionnally, configure your /etc/resolv.conf by editing it and adding:

nameserver 196.200.223.1

(you can also try  2001:470:933d::253 – it should work!)

2. NETSTAT

* Look at your routing table:

# netstat -rn

and

# netstat -rn -f inet6

-> What do you notice ?  Is the default gateway configured ?

-> How do you know ?

-> Is this true for both IPv4 and IPv6 ?

3. PING

* Let's ping the default gateway:

# ping 41.215.76.254

(Stop it with CTRL+C)

Try again with 2001:470:933d::254

* Let's ping something outside, on the Internet.  For example, www.afrinic.net

# ping www.afrinic.net

-> Do you get an answer ?

What about IPv6 ?

# ping6 2001:42d0::200:80:1

# ping6 www.afrinic.net

If not, check:

- that you have a gateway in IPv4 AND IPv6

- that you have an /etc/resolv.conf that contains a nameserver! (see 1.)

-> What do you notice about the response time (time=.. ms) ?

* Remove your default gateway:

# route delete default

# route delete -inet6 default

* Control that the default gateway is gone using the netstat -r, and netstat -r -f inet6, commands.

-> How can you be sure that the default gateway is no longer configured ?

* Now, try to ping:

- the local NOC machine:

# ping 41.215.76.253

# ping6 2001:470:933d::253

- www.afrinic.net:

# ping www.afrinic.net

# ping6 www.afrinic.net

- The IP address of www.afrinic.net

# ping 196.216.2.1

# ping6 2001:42d0::200:80:1

-> What do you observe ?

-> What is the consequence of removing the default gateway (in V4 and V6) ?

* Re-establish the default gateway:

# route add default 41.215.76.254

# route add -inet6 default 2001:470:933d::254

* Check that the default gateway is enabled again by pinging www.afrinic.net:

# ping www.afrinic.net

# ping6 www.afrinic.net

4. TRACEROUTE

* Traceroute to www.afrinic.net

# traceroute www.afrinic.net

# traceroute6 www.afrinic.net

* Try again, this time with the -n option:

# traceroute -n www.afrinic.net

# traceroute6 -n www.afrinic.net

-> Observe the difference with and without the '-n' option

5. ROUTE (IPv4 Example DO NOT DO THIS)

* Remove your default routes

# route delete default

* Add a route to the AfNOG backbone network through the gateway:

# route add 196.200.223.0/24 41.215.76.254

* Try to ping the backbone NOC:

# ping 196.200.223.1

* Try to ping www.afrinic.net:

# ping www.afrinic.net

* Try to ping 196.216.2.1:

# ping 196.216.2.1

-> What do you notice ?

-> What do you conclude ?

* Restore the default route:

# route add default 41.215.76.254

* Look at the routing table with the netstat -rn command:

# netstat -rn

-> What do you notice ?

-> Which route will be used to reach 196.200.223.1 ?

-> Which route will be used to reach 196.216.2.34 ?

* Let's imagine we have a network 10.10.10.0/24, which is reachable via

another router 41.215.76.250

-> What command would you type if you wanted to add this route to your

machine ?

6. TCPDUMP (IPv4 only)

* Run tcpdump on your system:

# tcpdump -n -i em0 icmp

(Note the use of the icmp keyword to limit viewing ICMP trafic)

* Ask someone else in the room to ping your machine, and look at your screen

* Delete the default route on your system:

# route delete default

* Repeat the ping

-> What do you notice ?